# MBG System Startup Checklist (Cold Boot / Fresh Start)

## 1. Hardware Setup

Plug the ESP32 into a USB port on your Windows 11 machine (use known working port).

Ensure any necessary sensors (e.g., DHT11, soil moisture) and relays are connected securely.

## 2. Start PlatformIO & Compile/Upload

Open VS Code.

Open your MBG project folder (e.g., C:\AIProjects\esp32_projects\My Balcony Gardener\).

Verify that platformio.ini points to correct COM3 (e.g., COM3).

Open main.cpp (v1.2.4) and verify it's the desired version (e.g., v1.2.4).

In the PlatformIO sidebar, click the checkmark to Build.

After successful build, click the right arrow (→) to Upload the sketch to ESP32.

Open the Serial Monitor via the plug icon and confirm ESP32 is booting and logging data.

## 3. Start Web Dashboard Locally

⚠️ Leave this PowerShell session open. It serves the local Vite dev server.

Open a PowerShell terminal (Win + X → Windows PowerShell).

cd into your frontend C:\AIProjects\esp32_projects\my-dashboard\ (e.g., cd C:\AIProjects\esp32_projects\my-dashboard\).

Run: npm run dev

Confirm you see something like:
VITE v4.x.x  ready in 300ms
➜  Local:   http://localhost:5173/

Keep this terminal open and running.

⚠️ Leave this PowerShell session open. It maintains the tunnel connection.

## 4. Start Cloudflared Tunnel (Remote Access)

Open a second PowerShell window.

cd into the folder with your C:\AIProjects\esp32_projects\cloudflared\ (e.g., cd C:\AIProjects\esp32_projects\cloudflared\).

Run: cloudflared tunnel run a97e498f-60a8-47f5-b03d-da43dfc488e (replace 'mbg' with your tunnel name, or use the --config file if applicable).

Wait for the output showing the public URL (e.g., https://mbg-dashboard.trycloudflare.com).

Open the link to verify the dashboard loads from the tunnel.

## 5. Verify Data Flow and System Status

On the dashboard, confirm:

- Temperature and humidity values are visible and updating.

- 'Last Watered' time is accurate.

- Logs are appearing correctly and in expected array format.

- Manual 'Water Now' button triggers watering.

If using logging and graphing, verify:

- Chart displays live data correctly.

- Duration and timestamps are readable.

## 6. Optional Development Tools

If debugging, leave Serial Monitor open and test real-time logs.

If using Git, confirm local repo status with:
git status

Use git commit to save changes once verified working.